No bug fixes, none found, but we did add some more debug

information on the last line of the screen and an additional

command line switch.

Command Line Switch: /Lxxxxxx.

/L is the serial LOCK rate and xxxxxx is the speed you are

locking the serial port at. This was added so those having

problems setting the COMx= can use an alternate method of passing

the lock rate.

E.g., /L38400 (Locked at 38400 baud)

Debug Info . . .

At the bottom of the screen, you'll find 4 windows; Uart Rate,

Connect Rate, Serial Port, and Flow Control.

Uart Rate:

This reflects the speed of the Uart or Serial port. This is based

on what YOU tell Tmodem the speed is with COMx=, /L, or /B

switch(es).

If you have a standard modem; 300, 1200, 2400 baud, the Uart Rate

should be the same as the connect rate. If it isn't, you are not

passing the correct parameters.

If you have a locked serial port, the Uart Rate should EQUAL the

lock rate. If it doesn't, then you are not passing the correct

parameters.

The Connect rate is what you pass in the /B parameter and if you

have a 300, 1200, or 2400 baud modem, the connect rate SHOULD

match the Uart rate. If it doesn't, you are not passing the

correct parameters.

If you have a locked serial port, the connect rate should NEVER

be the same as the Uart rate and it should never be higher than

9600 baud, the closest valid serial rate that any modem, to date,

runs at. Keeping in mind that 12000 and 14400 are not valid

serial rates and can not be used in the /B parameter.

If you have a locked serial port and connect with a 2400 baud

system and your Connect rate shows 9600 or higher, you're not

passing the correct connect rates and your transfers may not take

place and if they do, they may be slowed down by phase shifting.

Flow Control . . .

This shows you the TYPE of flow control Tmodem is using. By

default, it uses CTS (Hardware flow control).

Under Tmodem, Xon/Xoff just isn't fast enought to handle the data

flow.

If you do not have a 16550 Uart and you are locked at 19200 or

38400, you WILL be required to use the /F (RTS) command line

switch. This slows down the transfer, a bit, but this is better

than getting Uart RX overrun.