SF-LAN.DOC

Running SPITFIRE v3.5 on a LANtastic Network

SPITFIRE - Version 3.5

Copyright (C) by Mike Woltz, 1987-1994

Buffalo Creek Software

RUNNING SPITFIRE 3.5 ON A LANTASTIC NETWORK

-------------------------------------------

Running SPITFIRE v3.5 on a Local Area Network allows maximum speed as

well as capability of all multi-node strategies.  While hardware that

is required is considerable, LAN systems allow for performance not

possible using a multitasking system.

Information discussed in this section while specific to the LANtastic

Network is similar to other LAN systems such as Novell Netware.

Memory Issues:

-------------------

Several network drivers must be loaded depending upon the adapter

in use, whether the system is a server or a workstation, their

associated resources such as CD-ROM drives, etc. In addition, the

DOS SHARE program must be loaded on all workstations.  If the

machine is an 80386 or greater which has high memory available,

any or all of these drivers can be loaded there, freeing up

conventional memory. Many people report problems loading the

server program into high memory, but most other drivers will load

there with little difficulty.

Hardware Requirements:

---------------------

A Local Area Network contains at least one server and a workstation.

Some just contain servers, meaning that they all share resources.

While fast CPUs such as 80386 or i486 machines yield superior

performance, 8086 and 8088 based compatibles can be used.  In

addition, diskless workstations which boot from ROM then connect to

the Network can be utilized.

Each machine must have a Network card installed.  The 2MB/sec cards

made by LANtastic or Ethernet Compatible 10MB/sec cards can be used.

With proper drivers, Arcnet and Token Ring adapters will work too.

One 8 or 16 bit slot per CPU is required where appropriate.  Cables

and terminating resistors are supplied with the Network Startup Kits.

A LANtastic network is known as a "Peer-To-Peer Network". Another

popular type of network is known as a "Client Server Network."

The latter requires the server to be dedicated to honoring requests

from associated requester systems or Clients. It is possible to

set up LANtastic as a Client Server network by dedicating the server

through the use of the ALONE program or by just not running anything

else locally on the server. Servers such as this will run much

faster than those where local console activities are permitted.

Starting It All Up:

-------------------

LANtastic provides an excellent software installation program which

is utilized for each server and workstation on the Network.  After

installing all necessary hardware, the software must be installed.

If any nodes utilize the SPITFIRE printer option, the LANtastic

software must be configured accordingly so the printer can be

shared across the Network.  Setting redirector buffers to anything

other than the default may yield unpredictable printouts.  If the

default number of buffers cannot be used, experimentation may be

necessary to determine the correct number of buffers for the LAN.

Set up a separate SPITFIRE directory for each machine, and edit

each CONFIG.SYS and AUTOEXEC.BAT to both load all the LANtastic

programs used and to automatically boot SPITFIRE.  Set CONFIG.SYS

FILES= to the number of nodes multiplied by 25 files per node on

servers.  For example, if the Network has one server and three

workstations associated with it, and all have SPITFIRE nodes set

up on them, set FILES=100 on the server and FILES=25 on each

associated workstation.  These numbers might need to be increased

further when used with net-mail systems.

Workstations that boot when the associated server is still

booting or not available will wait until the server comes online.

This will occasionally yield the familiar "gleep" sound known by

anyone who has been near a LANtastic Network!  Once the server

finishes its boot routine, it will bring up its SPITFIRE Node and

all others on associated workstations will follow.

In the event that any machines on the Network use OS/2 instead of

DOS, it is important to remember that DOS does NOT recognize any

drive or partition that has been formatted with the OS/2 High

Performance File System (HPFS).  It is therefore recommended that

any HPFS partitions be the final drive on the system so that the

LAN will not have to "skip over" a drive that is not available.

If you have a CD-ROM drive on the network, be sure to load MSCDEX

after REDIR in autoexec.bat. Otherwise, the CD-ROM drive will not

be available to other nodes on the network.

Multitasking On A LAN:

----------------------

Each machine, whether a server or a workstation, can have at least

one node installed.  In order to install other nodes on the Network,

additional machines can be added.  At this time, DESQview can ONLY

be used on WORKSTATIONS!  Windows 3.1 can be used on both servers

and workstations providing LANtastic 4.0 or greater is in use.

Running DESQview on a LANtastic server will cause the system to

crash.  DESQview 386 and Windows 3.1 (in 386 Enhanced Mode) can be

successfully run on a workstation yielding as many nodes as memory

and available serial ports will allow.  Refer to the appropriate

documentation (SF-WIN.DOC or SF-DESQ.DOC) for running SPITFIRE under

DESQview and Windows for further information.